1sample

new

domain extent -1 1

wall generate box 0 0.85 0 0.85

ball distribute porosity 0.12 radius 3e-3 5e-3 box 0 0.85 0 0.85

ball attribute density 1800

ball attribute damp 0.1

cmat default model linear property kn 1e9 ks 1e9 fric 0.1

set timestep scale

cycle 1000 calm 50

set timestep auto

cycle 1000

solve aratio 1e-4

save sample

以上为了最初制样

2.bond

new

restore sample

contact model Linearpbond range contact type ball-ball

contact method deform emod 1e9 krat 1.5 range contact type ball-ball

contact method pb_deform emod 1e9 krat 1.5 range contact type ball-ball

contact property pb_rmul 1 lin_mode 1 range contact type ball-ball

contact property pb_ten 2e7 pb_coh 5e7 pb_fa 30 range contact type ball-ball

contact property fric 0.5 dp_nratio 0.2 range contact type ball-ball

contact method bond gap 1e-4

clean

ball delete range annulus center [0.85/2] [0.85/2] radius 0 [0.085/2]

contact property lin_force 0 0 lin_mode 1

cycle 1000 calm 100

solve

save bond

将接触模型重新设置

3.mesh

from fipy import CellVariable,Grid2D,Viewer,TransientTerm,DiffusionTerm,Gmsh2D

CellVariable：定义在单元中心上的变量

Grid2D：适用于规则网格的结构化二维网格生成

Viewer：Fipy的可视化工具

TransientTerm：代表PDE中的时间项

DiffusionTerm：代表PDE中的扩散/渗流项

Gmsh2D：支持复杂几何的非结构网络接口

from fipy.tools import numerix

import itasca

import numpy as np

import math

import fipy as fp

import itertools Python 内置的迭代器工具库

import itasca.ball as balls

################## create mesh

mesh = fp.Gmsh2D("""

cl__1 = 0.2;

Point(1) = {0, 0, 0, cl__1};

Point(2) = {0.85, 0, 0, cl__1};

Point(3) = {0.85, 0.85, 0, cl__1};

Point(4) = {0, 0.85, 0, cl__1};

Point(5) = {0.425, 0.425, -0, 0.07};

Point(6) = {0.3825, 0.425, -0, 0.07};

Point(7) = {0.4675, 0.425, -0, 0.07};

Line(1) = {4, 1,0.1};

Line(2) = {1, 2,0.1};

Line(3) = {2, 3,0.1};

Line(4) = {3, 4,0.1};

Circle(5) = {6, 5, 7};

Circle(6) = {7, 5, 6};

Line Loop(9) = {1, 2, 3, 4, -5, -6};

Plane Surface(9) = {9};

Physical Surface("face") = {9};

Physical Line("outlet") = {1, 2, 3, 4};

Physical Line("inlet") = {5,6};

"""% locals())

print("successful")

生成中间圆孔的网格

num_load = 0

kk = CellVariable(name='mobility',mesh=mesh, value=1e-6)

################## save mesh point

bs = 0.85*0.85-3.1415926*0.085*0.085/4挖去圆孔后的面积

m_radius = math.sqrt((bs/len(mesh.cellCenters[0]))/3.14)求出等效圆半径

with open('geo_information.txt','w') as f:

for ii in range(int(np.shape(mesh._cellVertexIDs.T)[0])):遍历每一个FiPy单元

id_ = ii + 1PFC中geometry/measure/group都不能用0作为ID

out1 = 'geometry set' + '  ' + str(id_)

f.write(out1 +'\n') 创建 geometry

out2 = 'geo polygon positions'

f.write(out2 +'  ') 定义多边形边界

for jj in range(3): 默认所有单元都是三角形

geo_posx = mesh.vertexCoords.T[int(mesh._cellVertexIDs.T[ii][jj])][0]

geo_posy = mesh.vertexCoords.T[int(mesh._cellVertexIDs.T[ii][jj])][1]

f.write( '(' + str(geo_posx)  + ',' +  str(geo_posy) + ')'  + '  ')

f.write('\n') 顶点坐标提取（非常底层）

out4 = 'measure create radius ' + str(m_radius) +' id ' + str(id_) + ' position ' + str(mesh.cellCenters[0][ii]) +' '+ str(mesh.cellCenters[1][ii])

f.write(out4+'\n') 在单元中心(cx, cy)创建一个半径为m_radius的测量对象

with open('group_information.txt','w') as f1:

for ii in range(int(np.shape(mesh._cellVertexIDs.T)[0])):

id_ = ii + 1

out3 = 'ball group' + '  ' + str(id_) +'  ' + 'range geometry' + '  ' + str(id_)+' '+ 'count odd'

f1.write(out3+'\n') 凡是落在第 i 个 FiPy 单元几何区域里的 ball但只取“奇数编号”的那一批

########Variable

kk = CellVariable(name='mobility',mesh=mesh, value=1e-6)

phi = CellVariable(name = "solution variable",mesh = mesh,value = 0.)

#####boundary

phi.constrain(0, mesh.physicalFaces["outlet"])

#phi.faceGrad.constrain(0., mesh.physicalFaces["edge"])

itasca.command("""

restore bond

call geo_information.txt

save mesh

""")

Solver.py

#

#itasca.command('''

#new

# ''')

#restore mesh

#call group_information.txt

for df in itasca.dfn.fracture.list():

xx = df.pos()[0]

yy = df.pos()[1]得到裂隙中心坐标

eel = []

eed = []

for ii in range(int(np.shape(mesh._cellVertexIDs.T)[0])):为裂隙寻找最近的FiPy单元

ex = mesh.cellCenters[0][ii]

ey = mesh.cellCenters[1][ii]

ll = math.sqrt((xx-ex)**2+(yy-ey)**2)

eel.append(ll)

xyz = min(eel) 找最近的 cell

xyzid = eel.index(xyz)

kk[xyzid] = kk[xyzid] + '1e-6'修改该单元的渗流参数

#ca.set_extra(1,kk.value.T)

#kk.value[int(ii)-1] = (B*ball_porosity**3*(2*mean_radius)**2/(1-ball_porosity)**2)

#

num_load = num_load  + 1

print(num_load) num_load = 加载步数 / 时间步 / 迭代计数，每循环一次：改变边界条件、重新解渗流场、重新施加渗流力

#eq = TransientTerm() == DiffusionTerm(coeff=kk)

phi.constrain(num_load*1e2 , mesh.physicalFaces["inlet"])施压逐步增大

eq = DiffusionTerm(coeff=kk) == 0.

eq.solve(var=phi)

#eq.solve(var=phi, dt=10000)

##########set  ball.extra value及添加渗流力

for b in balls.list():  遍历所有ball，施加流场变量

group_name = b.group()

if group_name != 'None':

b.set_extra(1,phi.value[int(group_name)-1])#水头

b.set_extra(2,phi.grad.value[0][int(group_name)-1])

b.set_extra(3,phi.grad.value[1][int(group_name)-1])

b.set_extra(5,phi.value[int(group_name)-1]*1e4)

#b.set_extra(4,phi.grad.value.T[int(group_name)-1]) #梯度矢量

#b.set_extra(5,-kk.value[int(group_name)-1]*phi.grad.value.T[int(group_name)-1])#流场

b.set_extra(6,kk.value[int(group_name)-1])#流场

ball_porosity= itasca.measure.Measure.porosity(itasca.measure.find(int(group_name)))#孔隙率

if ball_porosity > 0.7:

ball_porosity = 0.7防止：力无限放大、数值不稳定

force = -1e4*b.radius()*b.radius()*3.14*phi.grad.value.T[int(group_name)-1]/(1-ball_porosity)       Darcy 型体力 → 等效表面力

b.set_force_app(force)

for ii in range(int(np.shape(mesh._cellVertexIDs.T)[0])):

bp = balls.near((mesh.cellCenters[0][ii],mesh.cellCenters[1][ii]))

bp.set_extra(4,-kk.value[int(ii)]*phi.grad.value.T[int(ii)]) 对每个 FiPy cell：找最近的 DEM 球存储：𝑞=−𝑘∇𝜙（达西速度）

cal_kk.py

import itasca as it

##### 求解裂隙产生后渗透系数的变化

kk.setValue(1e-6)初始化后的渗透率

for df in it.dfn.fracture.list():遍历所有裂隙

xx = df.pos()[0]

yy = df.pos()[1]获取裂隙空间位置

eel = []

eed = []

for ii in range(int(np.shape(mesh._cellVertexIDs.T)[0])):遍历所有Fipy cell

ex = mesh.cellCenters[0][ii]

ey = mesh.cellCenters[0][ii]

ll = math.sqrt((xx-ex)**2+(yy-ey)**2)

eel.append(ll)裂隙中心到第i个cell中心的距离

xyz = min(eel)

xyzid = eel.index(xyz) 确定距离裂隙最近的cell编号

kk[xyzid] = kk[xyzid] + '1e-8'让裂隙所属cell渗透率增加

ca.set_extra(1,kk.value.T)

fracture

; fname: fracture.p3fis

;

; Simple environment to track fragmentation in a BPM.

; Track LinearPBond model "bond_change" events and turn them into fractures.

; Use Fragment logic and Ball Result logic to record fragemnt ids

;

;==============================================================================

define add_crack(entries)

local contact    = entries(1)

local mode       = entries(2)

local frac_pos   = contact.pos(contact)

local norm       = contact.normal(contact)

local dfn_label  = 'crack'

local frac_size

local bp1 = contact.end1(contact)

local bp2 = contact.end2(contact)

local ret = math.min(ball.radius(bp1),ball.radius(bp2));contact.method(contact,'pb_radius')

frac_size = ret

local inDir = vector(-comp.y(norm),comp.x(norm))

local vert1 = frac_pos + inDir * frac_size

local vert2 = frac_pos - inDir * frac_size

local arg = array.create(4)

arg(1) = 'vertices'

arg(2) = 2

arg(3) = vert1

arg(4) = vert2

crack_num = crack_num + 1

if mode = 1 then

; failed in tension

dfn_label = dfn_label + '_tension'

else if mode = 2 then

; failed in shear

dfn_label = dfn_label + '_shear'

endif

global dfn = dfn.find(dfn_label)

if dfn = null then

dfn = dfn.add(0,dfn_label)

endif

local fnew = dfn.addfracture(dfn,arg)

dfn.fracture.prop(fnew,'age')  = mech.age

dfn.fracture.extra(fnew,1) = bp1

dfn.fracture.extra(fnew,2) = bp2

crack_accum += 1

if crack_accum > 50

if frag_time < mech.age

frag_time = mech.age

crack_accum = 0

command

fragment compute

endcommand

; go through and update the fracture positions

loop for (local i = 0, i < 2, i = i + 1)

local name = 'crack_tension'

if i = 1

name = 'crack_shear'

endif

dfn = dfn.find(name)

if dfn # null

loop foreach local frac dfn.fracturelist(dfn)

local ball1 = dfn.fracture.extra(frac,1)

local ball2 = dfn.fracture.extra(frac,2)

if ball1 # null

if ball2 # null

local len = dfn.fracture.len(frac)/2.0

local pos = (ball.pos(ball1)+ball.pos(ball2))/2.0

if comp.x(pos)-len > xmin

if comp.x(pos)+len < xmax

if comp.y(pos)-len > ymin

if comp.y(pos)+len < ymax

dfn.fracture.pos(frac) = pos

endif

endif

endif

endif

endif

endif

endloop

endif

endloop

endif

endif

end

define track_init

command

set fish callback bond_break remove @add_crack

set fish callback bond_break @add_crack

endcommand

; reset global variables

global crack_accum = 0

global crack_num = 0

global track_time0 = mech.age

global frag_time = mech.age

global xmin = domain.min.x()

global ymin = domain.min.y()

global xmax = domain.max.x()

global ymax = domain.max.y()

end

;==============================================================================

; eof: fracture.p3fis

4.cal_load

new

restore mesh

ball attribute damp 0.5

ball attribute dis mul 0

ball attribute vel mul 0

;contact property lin_force 0 0 lin_mode 1

define identify_floaters  ;;;删除浮动颗粒

loop foreach local ball ball.list

ball.group.remove(ball,'floaters')

local contactmap = ball.contactmap(ball)

local size = map.size(contactmap)

if size <= 1 then

ball.group(ball) = 'floaters'

endif

endloop

end

@identify_floaters

ball delete range group 'floaters'

def show_water_head

loop foreach gs geom.set.list

gs_value = geom.set.find(gs)

v = geom.poly.pos(geom.poly.find(geom.set.find(gs),1))

geom.poly.extra(geom.poly.find(geom.set.find(gs),1) ,1) = ball.extra(ball.near(v),1)

endloop

end

;@show_water_head

def clear_extra

loop  foreach bp ball.list

ball.force.app(bp) = vector(0,0)

ball.group.remove(bp,ball.group(bp))

ball.extra(bp,1) = None

ball.extra(bp,2) = None

ball.extra(bp,3) = None

ball.extra(bp,4) = vector(0,0)

ball.extra(bp,5) = vector(0,0)

endloop

end

call fracture.p2fis

@track_init

call set_up.py

call solver.py

history hist_rep 500

history id 3 @crack_num

def loop_load

loop i(1,50)

show_water_head

command

cycle 500

ball delete range annulus center [0.85/2] [0.85/2] radius 0 [0.085/2]

endcommand

clear_extra

command

call group_information.txt

call solver.py

endcommand

endloop

end

@loop_load

save load